## Parameters – Overview model

## Mesh parameters

| Parameter | Value | Description |
| --- | --- | --- |
| Element size (ls) | 0.005 | Characteristic element length controlling mesh resolution |
| Mesh density (density) | 100 | N. of Nodes / unit length (lower = faster, higher = accurate) |
| Optimization (optimize) | True | Enables mesh quality optimization |
| Element order (order) | 2 | Quadratic tetrahedral elements (tet10) |

The mesh parameters were selected to ensure a balance between computational efficiency and numerical accuracy. A quadratic tetrahedral element formulation (tet10) was used, as higher-order elements are known to improve deformation accuracy in soft tissue simulations (Hsu et al., 2011).

The mesh density was set relatively low (density = 100) for initial simulations to reduce computation time during model development. Previous studies have shown that mesh refinement significantly affects displacement and stress accuracy, particularly in large deformation problems such as breast biomechanics (Samani et al., 2001; Chen et al., 2013). Therefore, this value represents a compromise between stability and computational cost, with the possibility of refinement in later stages.

## Geometry parameters

| Parameter | Value | Description |
| --- | --- | --- |
| Radius | 0.07 (m) | Overall breast size scaling parameter |
| Chest wall thickness | 0.002 (m) | Thickness of the fixed boundary region |
| Left ellipse pos. | 0.4 | Controls left-side curvature and glandular distribution |
| Right ellipse pos. | 0.05 | Controls nipple position |
| Center ellipse pos. | 0.3 | Controls vertical shape and curvature |

The breast geometry is defined using a simplified analytical representation, parameterized by a combination of circular and elliptical components. While this approach does not capture patient-specific anatomy, it provides a computationally efficient approximation of breast shape.

Similar simplified geometries have been widely used in early and intermediate finite element breast models (Samani et al., 2001; Mazier et al., 2023), particularly when the focus lies on understanding deformation behavior rather than patient-specific reconstruction. The chosen parameter values fall within realistic anatomical ranges reported in literature, where breast radii typically vary between 0.05 m and 0.15 m depending on patient variability (Ramião et al., 2016).

The elliptical parameters allow limited control over asymmetry and glandular distribution, which is an important aspect since real breast tissue is inherently non-uniform (Chen et al., 2024).

## Material properties

| Tissue | Density (kg/m³) | Bulk Modulus (Pa) | c1 (Pa) | c2 (Pa) | Description |
| --- | --- | --- | --- | --- | --- |
| Skin | 1100 | 480000 | 1200 | 1200 | Stiff outer layer |
| Adipose | 911 | 425000 | 109 | 106 | Soft fatty tissue |
| Glandular | 1041 | 425000 | 230 | 195 | Intermediate Stiffness tissue |

All tissues were modeled using a hyperelastic Mooney–Rivlin formulation, which is commonly applied in soft tissue biomechanics due to its ability to capture nonlinear large deformations (Eder et al., 2014; Hsu et al., 2011).

The relative stiffness hierarchy between tissues—skin being the stiffest, followed by glandular tissue, and adipose tissue being the softest—is consistent with experimental findings reported in literature (Ramião et al., 2016; Samani et al., 2007). The selected parameter values fall within commonly reported ranges and are based on previously validated FEM breast models.

Although more advanced material models such as Ogden formulations exist, the Mooney–Rivlin model provides a good balance between realism and computational efficiency, making it suitable for large-scale simulations and AI data generation.

## Tumor parameters

| Parameter | Value | Description |
| --- | --- | --- |
| Tumor enabled | False | Indicates if tumor is included |
| Density | 1079 (kg/m³) | Tumor density |
| Radius | 0.005 (m) | Tumor size |
| Position | [0.035, 0.04, 0] (m) | Spatial location |
| c1 (adipose) | 971 (Pa) | Stiffness when located in adipose tissue |
| c2 (adipose) | 939 (Pa) | Secondary stiffness parameter |
| c1 (glandular) | 920 (Pa) | Stiffness when located in glandular tissue |
| c2 (glandular) | 870 (Pa) | Secondary stiffness parameter |

The tumor is modeled as a spherical inclusion with increased stiffness relative to surrounding tissue. Although the tumor is disabled in the current simulations, its parameterization follows established modeling approaches where tumors are represented as localized regions with higher elastic coefficients (Ismail et al., 2018).

Literature consistently reports that malignant tumors exhibit significantly higher stiffness compared to healthy breast tissue, often by a factor of 5 to 20 (Samani et al., 2007). The selected parameters reflect this contrast, enabling future inclusion of tumor scenarios for validation or sensitivity studies.

## Simulation Control parameters

### Static Step (Gravity Initialization)

| Parameter | Value | Description |
| --- | --- | --- |
| Analysis type | STATIC | Static equilibrium step |
| Time step | 10 | N. of increments |
| Step size | 0.1 | Time increment size |

### Dynamic Step (Motion Simulation)

| Parameter | Value | Description |
| --- | --- | --- |
| Analysis type | DYNAMIC | Time-dependent simulation |
| Time step | 120 | N. of increments |
| Step size | 0.0025 | Time increment size |
| Max time step | 0.0025 | Upper bound for adaptive stepping |

The simulation is divided into two sequential steps: a static gravity loading phase followed by a dynamic motion phase. This approach is commonly used in breast biomechanics to establish an initial equilibrium configuration before applying dynamic loading conditions (Chen et al., 2013; Vavourakis et al., 2016).

The static step allows the model to settle under gravitational forces, which significantly influence breast deformation. The dynamic step then simulates motion through time-dependent loading, capturing inertia and nonlinear deformation effects.

The chosen time step sizes ensure numerical stability while maintaining reasonable computation times, as overly large time steps can lead to convergence issues in nonlinear simulations.

## Solver parameters

| Parameter | Value | Description |
| --- | --- | --- |
| Symmetry | symmetric | Stiffness matrix type |
| Equation scheme | staggered | Solver scheme |
| Line search tolerance | 0.9 | Convergence control |
| Max refinements | 15 | Maximum solver corrections |
| Displacement tolerance (dtol) | 0.001 | Convergence tolerance |
| Energy tolerance (etol) | 0.01 | Convergence tolerance |
| Beta | 0.5 | Time integration parameter |
| Gamma | 1.1 | Time integration parameter |

The solver settings are configured to ensure convergence in nonlinear large deformation simulations. A staggered solution scheme and symmetric stiffness matrix are used, which are commonly applied in soft tissue finite element analyses (FEBio documentation; Maas et al., 2012).

Convergence tolerances for displacement and energy were selected based on typical values reported in literature, ensuring a balance between numerical accuracy and computational cost. Time integration parameters (β and γ) were chosen to provide stable dynamic behavior, particularly for simulations involving rapid motion such as the applied parabolic jump.

## Loading and Motion parameters

| Parameter | Value | Description |
| --- | --- | --- |
| Gravity steps | 100 | Number of increments for gravity loading |
| Jump height | 0.01 | Maximum height of parabolic motion |
| Jump steps | 80 | Number of steps during motion |

The loading conditions consist of gravitational settling followed by a prescribed parabolic motion. Gravity plays a dominant role in breast deformation and must be accounted for to obtain realistic initial configurations (Vavourakis et al., 2016).

The dynamic motion is implemented as a simplified parabolic trajectory, representing vertical movement of the body. Similar simplified motion profiles have been used in previous studies to investigate breast dynamics without introducing overly complex boundary conditions (Chen et al., 2013; Yu et al., 2026).

The selected jump height remains small to avoid numerical instability while still inducing measurable deformation patterns.

## Output Settings

| Parameter | Value | Description |
| --- | --- | --- |
| Output format | VTK | Per time-step output files |
| Displacement output | True | Node displacements stored |
| Stress output | True | Cauchy stress stored |
| Relative volume | True | Volume change tracking |
| Extra variables | Multiple | Includes strain, material parameters, density |

Simulation outputs include nodal displacements, stress distributions, and relative volume changes. These quantities are commonly used to evaluate biomechanical behavior in FEM breast models (Mazier et al., 2023; Chen et al., 2024).

The use of VTK output files enables straightforward post-processing in Python-based tools such as PyVista, facilitating data extraction for further analysis and visualization. Additional variables, including strain and material parameters, are stored to allow detailed investigation of internal mechanical responses.

# Summary (for report)

The selected parameter set represents a balance between biomechanical realism and computational efficiency. While simplified geometric assumptions are used, the material properties and simulation approach are consistent with established finite element breast modeling studies.

By combining a hyperelastic material formulation, gravity-induced deformation, and dynamic motion, the model captures the key mechanical behaviors relevant for breast deformation analysis. This provides a suitable foundation for further refinement, particularly in terms of geometric realism and internal structural complexity, which are the primary focus of this project.

# Reference list

- Chen, L., et al. (2013). A study of breast motion using nonlinear dynamic finite element analysis.

- Chen, L., et al. (2024). Construction of multi-component finite element breast model.

- Eder, M., et al. (2014). Comparison of different material models for breast tissue simulation.

- Hsu, C. M., et al. (2011). Mechanical properties of human breast tissues.

- Ismail, M., et al. (2018). Finite element modeling for breast tumor detection.

- Maas, S. A., et al. (2012). FEBio: Finite elements for biomechanics.

- Mazier, A., et al. (2023). Image-based breast modeling pipeline.

- Ramião, N. G., et al. (2016). Mechanical properties of breast tissues: A review.

- Samani, A., et al. (2001). Biomechanical modeling of the human breast.

- Samani, A., et al. (2007). Elastic moduli of normal and pathological breast tissues.

- Vavourakis, V., et al. (2016). Inverse finite element analysis in breast modeling.

- Yu, H., et al. (2026). Dynamic breast deformation analysis.