# Workpackages overview (concept)

This is a list of workpackages for the research on the Early Warning Scan. Every partner is invited to select packages to contribute to. Tasks, inputs and outcomes can be added or moved to other work packages as desired.

# WP1 – Sensor & Acquisition

## Explanation:

The scanner is the frontend system used to register the chest area. It is adjustable in height and tilt position. The unit is designed to simultaniaously capture images using 52 cameras, from which 3D images are produced in post-processing using photogrammetry. To improve the accuracy of the 2D-to-3D conversion, projectors are used to project a guide pattern onto the chest. The maximum achievable number of frames per second is determined by the selected image sensors. With the current sensor OV9781, 45 frames per second is theoretically achievable.

## Tasks:

- Scanner development, calibration and validation

- Hardware evaluation

- Functional tests with volunteers

- ...

## Input:

- Scanner system to be validated

- Deliverables:

- Data aquisition system with validated specs

- Images: sets of 100 x 52 2D images

- ...

# WP2 - Markerless 4D Motion Capture & Correspondence

## Explanation:

Step 1 of processing the images from the scanner involves converting 2D images into 3D meshes. For this a technique called photogrammetry is used. This process is computational very intensive.

The resulting data is a series of 3D meshes (a 3D film), thus 4D information.

Because the individual meshes have no relationship to one another, they are markerless: not containing corresponding points

Given the large volume of data involved, this entire process needs to be automated.

## Tasks:

- 2D to 3D conversion and automation

- Photogrammetry software evaluation

- Literature review of techniques for quantifying breast surface displacement.

- Automation of the process

- ...

## Input:

- Series of 100 x 52 2D images of the breast movement (from WP1)

## Deliverables:

- 4D data (3D movie of the breast movement) as series of 3D meshes

- Preferred software choice

- ...

# WP3 - Generate synthetic dataset

## Explanation:

Since no data are available yet, we want to use artificial data to understand the behavior of a breast with and without a tumor.

FEM simulation is being used for this purpose. The aim is to be able to simulate both the shape and the movement of the breast, with and without a tumor.

## Tasks:

- Build simple and complex parameterizable FEM models of the breast and tumor

- Automated generating of geometries and movements

- ...

## Input:

- Hypothetical concepts of breast and tumor behaviour

- Multiple 3D samples of breast area

- Previous research

## Deliverables:

a dataset with positive and negative models that can be set in motion

...

# WP4 - Breast movement quantification

## Explanation:

To analyze chest movements, the 4D artificial data and the real-life data must be converted into a standardized format suitable for numerical processing (tensor). This format must be suitable as input for training an AI model.

This standardized format can also be used for research on which movements yield the best results.

## Tasks:

- Reseach on best approach for standardized measurement for AI input:

- EWS: displacement of breast surface in spherical coordinates

- Coen: inverse elastic reconstruction (also for validation of the artificial dataset)

- Analyse of effects of different movement stimuli

- ...

## Input:

- Artificial dataset

- Real-life dateset

## Deliverables:

- Standardized measuring method

- Known set of optimal breast movements

- ...

# WP5 - Data collection with volunteers

## Explanation:

Nothing beats real-life data. In this phase, the goal is to gather data from three groups of healthy volunteers:

- women who have not experienced any disruption of elasticity,
- women who have, for example due to minor or major surgical procedures,
- women who have a breast implant.

The goal is to:

- gain an understanding of the natural variation in the behavior of a healthy breast
- determine whether we can see a difference between a breast with and without an abnormality
- get an impression of whether implants pose an impediment

## Tasks:

- Technical research using validated scanner (WP1)

- Create dataset with healthy volunteers without anomalies

- Create dataset with healthy volunteers with anomalies (not cancer related)

- Create dataset with healthy volunteers with breast implants/reconstructions (exceptional group)

- Apply standardized measuring method from WP4 on data

- Apply optimal breast movements from WP4

- Refine negative breast data in artificial dataset with real-life data and re-train AI model

- ...

## Input:

- Number of volunteers to be screened (tbd)

## Deliverables:

- Real-life dataset from negative breast

- Conclusions about exclusion groups for future research

- Proof-of-principal: technical evidence that breasts with and without anomalies can be distinguished

- Evaluation of all previous WP's

- ...

# WP6 - Development AI model

## Explanation:

An AI model must be developed, trained and tested to classify the complex patterns of the 4D information.

## Tasks:

- Decide on AI model(s) to use

- Develop and train an AI model

- Partial validation of the model (only negative breasts from real-life dataset)

- Test the model

- ...

## Input:

- Artificial and real-life datasets in normalized format (from WP4)

## Deliverables

- Working AI model

- First impression of sensitivity and specifity of the system

- ...

# WP7 - Clinical research

## Explanation:

The dataset does not contain real-life data on breast cancer cases. To obtain this, it is necessary to gather data from diagnosed cases in a clinical setting. Permission from an Ethics Committee (METC) is required for this. By linking the established BI-RADS to the obtained scanner data, a labeled dataset is created that can be used to improve tumor simulation and/or directly expand the synthetic dataset and retrain the AI ​​model.

## Tasks:

- Obtain METC permission

- Collect data from patients BI-RADS0 - 6

- Determine the tumor behavior as a difference between negative and positive breast

- Refine data for positive breast model in artificial dataset with real-life data

- Re-train the AI model

- Redo validation of the AI model with positive and negative breasts

- ...

## Input:

- Volunteers with with suspected breast cancer

## Deliverables:

- Labeled dataset

- Knowledge of biomechanical tumor behavior

- Evidence that positive and negative can be distinguished to some degree

- Proof-of-concept

- ...

# WP8 - Clinical validation

## Explanation:

Clinical validation is required to determine sensitivity and specificity. For this purpose, a minimum number of participants must be determined and carefully selected to obtain the best possible indication.

## Tasks:

- Obtain METC permission

- Gather patient data

- Process patien data

## Input:

- Volunteers with with suspected breast cancer

- Control group without breast cancer

## Deliverables:

- Large labeled dataset

- A substantiated indication of sensitivity and specificity

# WP-A Data governance

## Explanation:

There are requirements regarding data processing, both in the area of ​​privacy and data security. Relevant are:

- AVG/GDPR
- WMO

## Tasks:

- Obtain METC permission

- Data Protection Impact Assessment (DPIA)

- Project-wide: guarantee data safety and privacy

- ...

## Input:

- Processes, procedures and systems to review

## Deliverables:

- Reviewed and approved data storage and usage

# WP-B Project management

## Explanation:

Project management ensures that an investigation remains feasible, organized, and compliant. Structure and planning are established, alongside the deployment of people and resources. Monitoring of progress and reporting.

## Tasks:

- Define roles and responsabilities

- Choose ICT tools

- Planning

- Financial monitoring

- ...

## Deliverables:

- Configured system for project management